PROJECT PROFILE FOR EXPANSION OF SPINNING MILL

(25200 SPINDLES)

- SCOPE OF THE PROJECT IN THE AREA OF OPERATION:

Sufficient information needs to be collected about the existing procedure for procurement of raw material from members/other sources and existing procedure for marketing and selling of cotton yarn. Also identify the market for surplus production.

The mill proposes to work 355 days in a year on 3 shift basis. The mill proposes to produce cotton yarn of count 20sK, 26sCH, 30sK, 30sCH, 34sK, 40sKH and 40sCH with average count of 30s. The Yarn production per spindle per shift on 40s conversion basis estimated at 105gms. Total yarn production after expansion is expected to be about 8738 MT per annum.

2.0       FOR IMPLEMENTATION OF THE PROJECT:

  - The society has to raise the requisite share capital;

  - Land purchase/leased by the society;

  - NOC from Pollution control board;

  - Prepared plan and estimates for civil work of main plant and ancillary buildings;

  - Obtained quotations from suppliers of plant & machinery required for modernization and

other equipments;

  - Obtain assurance from state electricity board for providing extra required electricity connections

3.0       PROJECT COMPONENTS AND THEIR TENTATIVE COST

(Rs. in Lakhs)

  - Land and Site Development                                                            00.00

  - Civil Works

- Main factory building for plant and machinery

(12000 sq.mtr)                                                                             840.00

- Ancilliary building (2000 Sq.M.)                                               120.00

- Humidification Plant (1400 Sq. M)                                              77.00

- Cotton & Yarn godowns (3000 Sq.M)                                       150.00

- Overhead & underground water tanks                                          50.00

- Cable Trenches, plumbing, sanitation, internal

Water supply etc.                                                                          13.00

TOTAL             1250.00

  - Plant  & Machinery

- Blow room with blendomat                                                        120.00

- Contamination control system                                                      50.00

- High production cards with can changer                                    470.00

- H.S. breaker draw Frame with double delivery                            60.00

- H.S. finisher draw frame with single delivery                            110.00

- Lap Former                                                                                   60.00

- Comber                                                                                       280.00

- Speed frame with overhead cleaner                                            240.00

- Ring Frames with  O.H.C.(1200 Spindles each)                         840.00

- Automatic cone winding machine                                            1000.00

- Yarn conditioning machine                                                           30.00

- AWCS for B.R. and cards                                                             35.00

- Stores & Spares                                                                           105.00

TOTAL              3400.00

  - Miscellaneous Fixed Assets

- Electrification                                                                              225.00

- Humidification Plant for ring frames

(Capacity 8 lakh CMH)                                                               235.00

- Compressor (2 Nos.)                                                                      50.00

- Fire fighting equipment                                                                 20.00

- Card room and cot room accessories                                             25.00

- Uster Eveness Tester                                                                     70.00

- Classimat Tester                                                                            35.00

TOTAL               660.00

3.5       Pre operative expenses                                                                       40.00

3.6       Margin money for working capital requirement                           650.00

TOTAL PROJECT COST                                     6000.00

- PROJECT IMPLEMENTATION SCHEDULE:

It is expected that the project is completed within one and half year after sanction of final assistance.

- REQUIREMENT OF STAFF:

The mill will require 648 workers and 15 staff.

- PATTERN OF FUNDING:

As per NCDC’s pattern of Funding for cooperatively developed states 90% of loan is sanctioned to the state Govt. for passing on to the beneficiary societies as 60% loan and 30% share capital contribution. For cooperatively under developed states and cooperatively least developed states 92.5% of loan is sanctioned to the state govt. for passing on to the beneficiary societies as 60% loan and 32.5% share capital.

Loan application forms are available with the office of Registrars of cooperative societies/Regional Directorates of the NCDC besides Head Office at New Delhi. Application forms can also be downloaded from NCDC website(http://ncdc.in).